# HEMP PROFICIENCY TESTING GENERAL INSTRUCTIONS FOR SAMPLES HM20SEP-1 and HM20SEP-2

Results Due October 15th, 2020

- The samples provided are ready for analysis. Do not perform any preliminary processing step such as drying or grinding prior to analysis. The sample number syntax is: HM20SEP-1 (material, year, sample month, sample number).

- Provide triplicate analyte results for each sample. Analyze each sample replicate on different days. Analytes with a single result will not be considered in the statistical analysis.

- Analyte concentrations are reported on an “as-received” or “dry weight” basis. As-Received is the concentration of the sample without any drying. To report on a Dry-Weight basis you can determine the moisture content of the sample and calculate the concentration based on "dry weight" as shown below. You cannot report on a dry weight basis unless you determine the moisture content of the sample. You may report concentrations on both “as-received” and “dry weight” basis.

% dry weight basis = % as-received x (100 / (100 - % moisture) )

- Instructions for submitting data can be found at http://www.rs.uky.edu/regulatory/hpt/submit.php.	There is an expanded list of methods and analytes for data submission including additional cannabinoids and metals.

- Record your analyte result to 4 decimal places with the appropriate method code. Record the result with the “Other” method if your result is from a method not shown.

- Very low concentrations are normally reported as less than a limit of detection (LOD) or limit of quantification (LOQ). Record very low concentrations as you would report results to your client. Place a "<" in front of the value considered a limit in your lab. For example, record "<0.0100" for a limit of 0.01.

- Contact Frank Sikora (fsikora@uky.edu) if you have questions on what method your results should be reported as.

- Contacts:

Frank Sikora fsikora@uky.edu 859-218-2452

Marilyn Smith mm.smith@uky.edu 859-218-2468