# Hemp PT Certificate of Analysis

## Hemp PT Sample HM20SEP-1 Material: hemp

± Standard

| Analyte | Value | Uncertainty | # Labs |
| --- | --- | --- | --- |
|  | % as received |  |  |
| Δ9-THC | 0.07928 | 0.00201 | 59 |
| Δ9-THCA | 0.08381 | 0.00216 | 56 |
| CBD | 1.808 | 0.038 | 52 |
| CBDA | 3.763 | 0.085 | 50 |
| CBN | 0.01307 | 0.00113 | 28 |
| Total Δ9-THC | 0.1558 | 0.0032 | 62 |
| Total CBD | 5.110 | 0.107 | 50 |
| CBDVA | 0.02834 | 0.00853 | 6 |
| CBG | 0.03746 | 0.00484 | 18 |
| CBGA | 0.05451 | 0.00343 | 21 |
| CBC | 0.1174 | 0.0025 | 26 |
| Moisture | 11.38 | 0.44 | 19 |
|  | % dry weight |  |  |
| Δ9-THC | 0.09127 | 0.00338 | 20 |
| Δ9-THCA | 0.09226 | 0.00431 | 19 |
| CBD | 2.021 | 0.082 | 17 |
| CBDA | 4.186 | 0.213 | 16 |
| CBN | 0.02033 | 0.00521 | 8 |
| Total Δ9-THC | 0.1705 | 0.0057 | 23 |
| Total CBD | 5.715 | 0.216 | 19 |
| CBGA | 0.06832 | 0.00848 | 6 |
| CBC | 0.1276 | 0.0055 | 9 |

## Hemp PT Sample HM20SEP-1 Material: hemp

| Analyte | Value | ± Standard Uncertainty | # Labs |  |
| --- | --- | --- | --- | --- |
| ug/g as-received |  |  |  |  |
| Cd | 0.07341 | 0.00123 | 8 |  |
| As | 0.06865 | 0.00883 | 7 |  |
| Pb | 1.450 | 0.101 | 9 |  |
| Cr | 0.7566 | 0.1509 | 6 |  |
| Zn | 100.1 | 4.6 | 6 |  |

- Data obtained from participant results in the University of Kentucky Hemp PT Program.

- Sample was prepared on 9/6/20. Stored at room temperature until 10/15/20. Stored at -20 C after 10/15/20.

- Values are robust means. Standard uncertainty determined as 1.25 x robust standard deviation / sqrt(#Labs). Results only shown for analytes tested by 6 or more labs.