# Hemp PT Certificate of Analysis

## Hemp PT Sample HM20SEP-2 Material: hemp

± Standard

| Analyte | Value | Uncertainty | # Labs |
| --- | --- | --- | --- |
|  | % as received |  |  |
| Δ9-THC | 0.2311 | 0.0042 | 61 |
| Δ9-THCA | 0.05979 | 0.00230 | 54 |
| CBD | 3.690 | 0.076 | 52 |
| CBDA | 3.801 | 0.070 | 50 |
| CBN | 0.02864 | 0.00132 | 36 |
| Total Δ9-THC | 0.2858 | 0.0058 | 64 |
| Total CBD | 7.031 | 0.135 | 50 |
| CBDV | 0.03922 | 0.00808 | 12 |
| CBDVA | 0.04218 | 0.00434 | 10 |
| CBG | 0.08122 | 0.00609 | 23 |
| CBGA | 0.09885 | 0.00454 | 25 |
| THCV | 0.02033 | 0.00591 | 6 |
| CBC | 0.2032 | 0.0044 | 26 |
| Moisture | 7.085 | 0.406 | 19 |

## Hemp PT Sample HM20SEP-2 Material: hemp

Analyte	Value

± Standard

Uncertainty	# Labs

% dry weight

| Δ9-THC | 0.2514 | 0.0075 | 20 |
| --- | --- | --- | --- |
| Δ9-THCA | 0.06301 | 0.00354 | 18 |
| CBD | 3.888 | 0.128 | 17 |
| CBDA | 4.113 | 0.149 | 16 |
| CBN | 0.03540 | 0.00440 | 11 |
| Total Δ9-THC | 0.3065 | 0.0091 | 23 |
| Total CBD | 7.520 | 0.264 | 19 |
| CBG | 0.07903 | 0.01322 | 7 |
| CBGA | 0.1149 | 0.0133 | 6 |
| CBC | 0.2163 | 0.0082 | 9 |
| Cd | ug/g 0.1650 | as-received 0.0067 | 9 |
| As | 0.3657 | 0.0349 | 9 |
| Pb | 1.063 | 0.063 | 9 |
| Ni | 4.091 | 0.407 | 6 |
| Cr | 9.419 | 2.670 | 6 |
| Zn | 53.34 | 4.32 | 6 |

- Data obtained from participant results in the University of Kentucky Hemp PT Program.

- Sample was prepared on 9/6/20. Stored at room temperature until 10/15/20. Stored at -20 C after 10/15/20.

- Values are robust means. Standard uncertainty determined as 1.25 x robust standard deviation / sqrt(#Labs). Results only shown for analytes tested by 6 or more labs.