# Hemp PT Certificate of Analysis

## Hemp PT Sample HM22NOV-1 Material: hemp

± Standard

| Analyte | Value | Uncertainty | # Labs |
| --- | --- | --- | --- |
|  | % as received |  |  |
| Δ9-THC | 0.1920 | 0.0047 | 40 |
| Δ9-THCA | 0.06973 | 0.00207 | 37 |
| CBD | 1.835 | 0.033 | 31 |
| CBDA | 0.9960 | 0.0317 | 28 |
| CBN | 0.05886 | 0.00258 | 23 |
| Total Δ9-THC | 0.2565 | 0.0071 | 47 |
| Total CBD | 2.682 | 0.051 | 33 |
| CBG | 0.7956 | 0.0218 | 15 |
| CBGA | 1.214 | 0.023 | 13 |
| CBC | 0.1853 | 0.0055 | 15 |
| Moisture | 6.656 | 0.463 | 22 |
|  | % dry weight |  |  |
| Δ9-THC | 0.1961 | 0.0128 | 13 |
| Δ9-THCA | 0.07698 | 0.00682 | 12 |
| CBD | 1.996 | 0.043 | 12 |
| CBDA | 1.037 | 0.054 | 11 |
| CBN | 0.06097 | 0.00379 | 12 |
| Total Δ9-THC | 0.2582 | 0.0145 | 18 |
| Total CBD | 2.946 | 0.086 | 14 |
| CBG | 0.8835 | 0.0170 | 6 |
| CBC | 0.1986 | 0.0087 | 7 |

## Hemp PT Sample HM22NOV-1 Material: hemp

| Analyte | Value | ± Standard Uncertainty | # Labs |  |
| --- | --- | --- | --- | --- |
| ug/g as-received |  |  |  |  |
| Cd | 0.1262 | 0.0099 | 7 |  |
| As | 0.09810 | 0.00898 | 7 |  |
| Pb | 0.6152 | 0.0437 | 9 |  |

- Data obtained from participant results in the University of Kentucky Hemp PT Program.

- Sample was shipped on 11/8/22. Stored at room temperature until 12/21/22. Stored at -20 C after 12/21/22.

- Values are robust means. Standard uncertainty determined as 1.25 x robust standard deviation / sqrt(#Labs). Results only shown for analytes tested by 6 or more labs.