# Hemp PT Certificate of Analysis

## Hemp PT Sample HM24NOV-2 Material: hemp

± Standard

| Analyte | Value | Uncertainty | # Labs |
| --- | --- | --- | --- |
|  | % as received |  |  |
| Δ9-THC | 0.1983 | 0.0066 | 29 |
| Δ9-THCA | 0.05212 | 0.00227 | 27 |
| CBD | 4.827 | 0.124 | 23 |
| CBDA | 3.795 | 0.083 | 22 |
| CBN | 0.05046 | 0.00315 | 20 |
| Total Δ9-THC | 0.2460 | 0.0078 | 36 |
| Total CBD | 8.295 | 0.138 | 22 |
| CBDV | 0.03902 | 0.00799 | 6 |
| CBG | 0.2253 | 0.0196 | 10 |
| CBGA | 0.1965 | 0.0120 | 10 |
| CBC | 0.2368 | 0.0122 | 8 |
| Moisture | 6.903 | 0.713 | 15 |
|  | % dry weight |  |  |
| Δ9-THC | 0.2246 | 0.0078 | 8 |
| Δ9-THCA | 0.05725 | 0.00249 | 7 |
| CBD | 5.426 | 0.055 | 6 |
| CBDA | 4.183 | 0.167 | 6 |
| CBN | 0.05439 | 0.00319 | 7 |
| Total Δ9-THC | 0.2570 | 0.0102 | 14 |
| Total CBD | 8.964 | 0.332 | 10 |
|  | ug/g as-received |  |  |
| Cd | 0.1426 | 0.0053 | 7 |
| Pb | 0.1651 | 0.0084 | 7 |

## Hemp PT Sample HM24NOV-2 Material: hemp

Analyte	Value

± Standard

Uncertainty	# Labs

- Data obtained from participant results in the University of Kentucky Hemp PT Program.

- Sample was shipped on 11/11/24. Stored at room temperature until 12/20/24. Stored at -20 C after 12/20/24.

- Values are robust means. Standard uncertainty determined as 1.25 x robust standard deviation / sqrt(#Labs). Results only shown for analytes tested by 6 or more labs.