# Hemp PT Certificate of Analysis

## Hemp PT Sample HM25SEP-1 Material: Hemp

± Standard

| Analyte | Value | Uncertainty | # Labs |
| --- | --- | --- | --- |
|  | % as received |  |  |
| Δ9-THC | 0.2144 | 0.0057 | 30 |
| Δ9-THCA | 0.03292 | 0.00213 | 22 |
| CBD | 5.634 | 0.177 | 22 |
| CBDA | 2.662 | 0.113 | 22 |
| CBN | 0.0620 | 0.00192 | 20 |
| Total Δ9-THC | 0.2459 | 0.0068 | 32 |
| Total CBD | 8.209 | 0.236 | 24 |
| CBDV | 0.05083 | 0.00457 | 7 |
| CBG | 0.1727 | 0.0111 | 8 |
| CBGA | 0.1350 | 0.0116 | 9 |
| CBC | 0.2367 | 0.0148 | 9 |
| Moisture | 7.167 | 0.590 | 13 |
|  | % dry weight |  |  |
| Δ9-THC | 0.2312 | 0.0104 | 9 |
| Δ9-THCA | 0.03414 | 0.00667 | 6 |
| CBD | 6.277 | 0.279 | 7 |
| CBDA | 2.820 | 0.191 | 7 |
| CBN | 0.06765 | 0.00294 | 9 |
| Total Δ9-THC | 0.2538 | 0.0117 | 13 |
| Total CBD | 8.617 | 0.50 | 10 |
|  | ug/g as-received |  |  |
| Cd | 0.1795 | 0.010 | 6 |
| Pb | 0.2520 | 0.0211 | 7 |

## Hemp PT Sample HM25SEP-1 Material: Hemp

Analyte	Value

± Standard

Uncertainty	# Labs

- Data obtained from participant results in the University of Kentucky Hemp PT Program.

- Sample was shipped on 09/16/25. Stored at room temperature until 10/30/25. Stored at -20 C after 10/30/25.

- Values are robust means. Standard uncertainty determined as 1.25 x robust standard deviation / sqrt(#Labs). Results only shown for analytes tested by 6 or more labs.