# Hemp PT Certificate of Analysis

## Hemp PT Sample HM25SEP-2 Material: Hemp

± Standard

| Analyte | Value | Uncertainty | # Labs |
| --- | --- | --- | --- |
|  | % as received |  |  |
| Δ9-THC | 0.1051 | 0.0036 | 30 |
| Δ9-THCA | 0.01497 | 0.00320 | 17 |
| CBD | 5.186 | 0.176 | 22 |
| CBDA | 1.654 | 0.079 | 21 |
| CBN | 0.09213 | 0.00390 | 20 |
| Total Δ9-THC | 0.1197 | 0.0047 | 32 |
| Total CBD | 6.855 | 0.236 | 24 |
| CBDV | 0.05028 | 0.00479 | 7 |
| CBG | 0.2831 | 0.020 | 9 |
| CBGA | 0.1474 | 0.0121 | 9 |
| CBC | 0.2268 | 0.0144 | 9 |
| Moisture | 7.013 | 0.549 | 13 |
|  | % dry weight |  |  |
| Δ9-THC | 0.1163 | 0.0047 | 9 |
| CBD | 5.635 | 0.282 | 7 |
| CBDA | 1.754 | 0.153 | 7 |
| CBN | 0.09773 | 0.00561 | 9 |
| Total Δ9-THC | 0.1254 | 0.0045 | 13 |
| Total CBD | 7.197 | 0.414 | 10 |
|  | ug/g as-received |  |  |
| Cd | 0.1785 | 0.0104 | 6 |
| Pb | 0.3581 | 0.0599 | 7 |

## Hemp PT Sample HM25SEP-2 Material: Hemp

Analyte	Value

± Standard

Uncertainty	# Labs

- Data obtained from participant results in the University of Kentucky Hemp PT Program.

- Sample was shipped on 09/16/25. Stored at room temperature until 10/30/25. Stored at -20 C after 10/30/25.

- Values are robust means. Standard uncertainty determined as 1.25 x robust standard deviation / sqrt(#Labs). Results only shown for analytes tested by 6 or more labs.